Magnet Types

2. Electromagnets

B/ Superconducting Magnets

- Superconducting magnets are Today's most commonly used in MRIs.

- These superconductors, such as niobium-tin and niobium-titanium are used to make the coil windings for superconducting magnets.

- The magnetic field is generated by a passing electrical current through coils of wire. The wire is surrounded with a coolant, such as liquid helium, to reduce the electric resistance of the wire.

- At 4 Kelvin (-269 C) electric wire looses its resistance. Once a system is energized, it won't loose its magnetic field.

- Superconductivity allows for systems with very high field strengths up to 12 Tesla.

- The ones that are most used in clinical environments run at 1.5 Tesla. Most superconducting magnets are bore type magnets.

Figure (1) : Superconducting Magnets

- A number of vacuum vessels, which act as temperature shields, surround the core. These shields are necessary to prevent the helium to boil off too quickly.

- Another advantage of superconducting magnets is the high magnetic field homogeneity.

Figure (2) : bore type magnets.

ADVANTAGES

- High field strength

- High field homogeneity

- Low power consumption

- High SNR (SNR: The ratio of signal power to the noise power)

- Fast scanning

DISADVANTAGES

- High capital costs

- High cryogen costs

- Acoustic noise

- Motion artifacts

- Technical complexity